Design & Technology

AQA A-Level

Design communication

Materials required for questions

- Pencil

- Rubber

- Calculator

Instructions

- Use black ink or ball-point pen

- Try answer all questions

- Use the space provided to answer questions

- Calculators can be used if necessary

- For the multiple choice questions, circle your answer

Advice

- Marks for each question are in brackets

- Read each question fully

- Try to answer every question

- Don’t spend too much time on one question

Good luck!

Q1. Which technique is most effective for presenting statistical data trends to clients?

A		3D sketching

B		Graphs

C		Report writing

Q2. What is the primary purpose of rendering in a design drawing?

A		To add precise dimensions

B		To enhance visual realism using colour/texture

C		To create technical tables

Q3. Which method ensures components are manufactured to exact specifications?

A		Mixed media collage

B		2D abstract sketching

C		Dimensioning with tolerances

Q4. A designer uses isometric and orthographic views in a presentation. This demonstrates:

A		Use of 2D/3D sketching

B		Report writing standards

C		Table formatting

Q5. Evaluate the following techniques for rendering a design:

- using computer aided design (CAD)

- hand generated 								(6 marks)

Q6. The images below show two different design communication techniques. Discuss why a designer may use each technique to communicate information (6 marks)

Answers

Q1. B

Q2. B

Q3. C

Q4. A

Q5.

CAD Rendering

- Costly software and powerful computer processors are needed which can make it prohibitive.

- A high degree of competency in the use of the CAD software is needed to fully exploit all functions which may require lengthy training.

- Software may require purchased credits to perform high quality renders adding to the cost of the technique.

- Photo realistic renders are feasible.

- A huge range of material textures are available.

- Light sources and shadow can be represented.

- Rendered images can be placed into scenarios.

- CAD can be emailed/shared for instant feedback from clients/focus groups.

- CAD renders can be easily edited.

Hand rendering

- A great deal of skill is needed to proficiently achieve a high quality render using markers, coloured pencils or inks etc.

- Specialist paper is needed to avoid the colours running.

- A hand rendered drawing can be time consuming to complete and takes longer than CAD.

- Tone and shadow can be achieved through a wide range of available colours.

- Flat smooth colours can be achieved with the use of a blender.

- Minimal specialist equipment is needed making the process more affordable than CAD.

- Hard to edit the render and usually would require restarting

Q6.

Exploded view:

- Designers may use exploded views to produce assembly instruction booklets for flat pack furniture to assist the consumer.

- Exploded views allow the viewer to see all components within a product clearly.

- Exploded views can be used on assembly lines to assist during production.

- Exploded views may be used to communication information on internal assemblies to a client during a design meeting.

- Using CAD software allows a designer to create an exploded view on screen and re-assemble a product virtually when working with a client so all components can be seen.

- Exploded views allow consumers to identify and order replacement parts.

Sectional view (Orthographic):

- Sectional views allow the viewer to see internal and hidden details within an assembly.

- Using 2D sectional views allows dimensions of hidden components to be added onto engineering drawings.

- Sectional views allow designers to visualise the interaction between separate hidden components.